嵌入式系统原理与技术   测控2022-01班  赵成鲁  2022110863     第十一章作业

- 嵌入式上机作业

- 题目要求

    - 二、 硬件电路与接线表

1、硬件电路

图1  流水灯模块原理图及连接原理图

图2 按键原理图

2、硬件接线表

| M-STM32F407xx | 流水灯模块 |
| --- | --- |
| PA2 | LED1 |
| PC2 | LED2 |
| PA4 | LED3 |
| PA7 | LED4 |
| PC3 | LED5 |
| PA3 | LED6 |
| PA5 | LED7 |
| PC4 | LED8 |
| 3.3V | VCC |

    - 三、软件设计方案

按键采用中断触发，在中断服务程序中设置标志位，实现不同流水灯的标志位切换。

流水灯采用非阻塞式写法，避免单片机卡在子程序中无法及时响应流水灯标志位的切换。采用定时器中断实现对计数变量自增计时，在对应时间控制LED的亮灭实现流水灯。

    - 四、实验结果

图1  (1)初始化：LED全灭

图2  (2) 按第一次：LED全亮

图3  (3)按第二次：L0~L7：流水灯（间隔500ms）

图4  (4) 按第三次：L0~L7闪烁（间隔500ms）

    - 五、附录：程序（自己设计的程序，包括底层自己设计的程序，库函数不需要附录）

main.c

#include "stm32f4xx.h"

#include "LED.h"

#include "KEY.h"

#include "exti.h"

#include "timer.h"

extern u8 key_state;

int main(void)

{

TIM3_Int_Init(1000-1,8400-1);

LED_Init();

KEY_Init();

EXTIx_Init();

while(1)

{

}

}

void TIM3_IRQHandler(void)

{

if(TIM_GetITStatus(TIM3,TIM_IT_Update)==SET)

{

LED_Tick(key_state);

TIM_ClearITPendingBit(TIM3,TIM_IT_Update);

}

}

LED.c

#include "stm32f4xx.h"

#include <stdbool.h>

u32 led_count=0;

void LED_Init(void)

{

GPIO_InitTypeDef GPIOA_InitStructure;

GPIO_InitTypeDef GPIOC_InitStructure;    //放在这里的原因是 不允许初始化变量在函数的中间

RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOA,ENABLE);

GPIOA_InitStructure.GPIO_Mode = GPIO_Mode_OUT;

GPIOA_InitStructure.GPIO_OType = GPIO_OType_PP;

GPIOA_InitStructure.GPIO_Pin = (GPIO_Pin_2|GPIO_Pin_3|GPIO_Pin_4|GPIO_Pin_5|GPIO_Pin_7);

GPIOA_InitStructure.GPIO_PuPd = GPIO_PuPd_UP;

GPIOA_InitStructure.GPIO_Speed = GPIO_Speed_2MHz;

GPIO_Init(GPIOA,&GPIOA_InitStructure);

RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOC,ENABLE);

GPIOC_InitStructure.GPIO_Mode = GPIO_Mode_OUT;

GPIOC_InitStructure.GPIO_OType = GPIO_OType_PP;

GPIOC_InitStructure.GPIO_Pin = (GPIO_Pin_2|GPIO_Pin_3|GPIO_Pin_4);

GPIOC_InitStructure.GPIO_PuPd = GPIO_PuPd_UP;

GPIOC_InitStructure.GPIO_Speed = GPIO_Speed_2MHz;

GPIO_Init(GPIOC,&GPIOC_InitStructure);

GPIO_SetBits(GPIOA,GPIO_Pin_2|GPIO_Pin_3|GPIO_Pin_4|GPIO_Pin_5|GPIO_Pin_7);

GPIO_SetBits(GPIOC,GPIO_Pin_2|GPIO_Pin_3|GPIO_Pin_4);

}

void LED_SET(int led_num,bool led_state)

{

switch(led_num)

{

case 1:

{

if(led_state==false)

GPIO_SetBits(GPIOA,GPIO_Pin_2);

else

GPIO_ResetBits(GPIOA,GPIO_Pin_2);

break;

}

case 2:

{

if(led_state==false)

GPIO_SetBits(GPIOC,GPIO_Pin_2);

else

GPIO_ResetBits(GPIOC,GPIO_Pin_2);

break;

}

case 3:

{

if(led_state==false)

GPIO_SetBits(GPIOA,GPIO_Pin_4);

else

GPIO_ResetBits(GPIOA,GPIO_Pin_4);

break;

}

case 4:

{

if(led_state==false)

GPIO_SetBits(GPIOA,GPIO_Pin_7);

else

GPIO_ResetBits(GPIOA,GPIO_Pin_7);

break;

}

case 5:

{

if(led_state==false)

GPIO_SetBits(GPIOC,GPIO_Pin_3);

else

GPIO_ResetBits(GPIOC,GPIO_Pin_3);

break;

}

case 6:

{

if(led_state==false)

GPIO_SetBits(GPIOA,GPIO_Pin_3);

else

GPIO_ResetBits(GPIOA,GPIO_Pin_3);

break;

}

case 7:

{

if(led_state==false)

GPIO_SetBits(GPIOA,GPIO_Pin_5);

else

GPIO_ResetBits(GPIOA,GPIO_Pin_5);

break;

}

case 8:

{

if(led_state==false)

GPIO_SetBits(GPIOC,GPIO_Pin_4);

else

GPIO_ResetBits(GPIOC,GPIO_Pin_4);

break;

}

}

}

void LED_Tick(u8 key_state)

{

switch(key_state)

{

case 1:

LED_SET(1,true);LED_SET(2,true);LED_SET(3,true);LED_SET(4,true);LED_SET(5,true);LED_SET(6,true);LED_SET(7,true);LED_SET(8,true);

break;

case 2:

if(led_count==45)

{

led_count=0;

}

if(led_count%5==0)

{

LED_SET(1,false);LED_SET(2,false);LED_SET(3,false);LED_SET(4,false);LED_SET(5,false);LED_SET(6,false);LED_SET(7,false);LED_SET(8,false);

LED_SET(led_count/5,true);

}break;

case 3:

if(led_count==10)

{

led_count=0;

}

switch(led_count)

{

case 0:LED_SET(1,false);LED_SET(2,false);LED_SET(3,false);LED_SET(4,false);LED_SET(5,false);LED_SET(6,false);LED_SET(7,false);LED_SET(8,false);break;

case 5:LED_SET(1,true);LED_SET(2,true);LED_SET(3,true);LED_SET(4,true);LED_SET(5,true);LED_SET(6,true);LED_SET(7,true);LED_SET(8,true);break;

}break;

}

led_count ++;

}

LED.h

#ifndef __LED_H

#define __LED_H

#include "stm32f4xx.h"

#include <stdbool.h>

void LED_Init(void);

void LED_SET(int led_num,bool led_state);

void LED_Tick(u8 key_state);

#endif

KEY.c

#include <stm32f4xx.h>

#include "delay.h"

#include "key.h"

void KEY_Init(void)

{

GPIO_InitTypeDef  GPIO_InitStructure;

RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOA|RCC_AHB1Periph_GPIOE, ENABLE);

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_4;

GPIO_InitStructure.GPIO_Mode = GPIO_Mode_IN;

GPIO_InitStructure.GPIO_Speed = GPIO_Speed_100MHz;

GPIO_InitStructure.GPIO_PuPd = GPIO_PuPd_DOWN ;

GPIO_Init(GPIOE, &GPIO_InitStructure);

GPIO_InitStructure.GPIO_Pin = GPIO_Pin_0;

GPIO_InitStructure.GPIO_PuPd = GPIO_PuPd_DOWN ;

GPIO_Init(GPIOA, &GPIO_InitStructure);

}

KEY.h

#ifndef __KEY_H

#define __KEY_H

#include <stm32f4xx.h>

#define KEY0 		GPIO_ReadInputDataBit(GPIOE,GPIO_Pin_4) //PE4

#define WK_UP 		GPIO_ReadInputDataBit(GPIOA,GPIO_Pin_0)	//PA0

#define KEY0_PRES 1

#define WKUP_PRES 2

void KEY_Init(void);

#endif

exit.c

#include "exti.h"

#include <stm32f4xx.h>

u8 key_state;

extern u32 led_count;

void EXTIx_Init(void)

{

NVIC_InitTypeDef   NVIC_InitStructure;

EXTI_InitTypeDef   EXTI_InitStructure;

RCC_APB2PeriphClockCmd(RCC_APB2Periph_SYSCFG, ENABLE);

SYSCFG_EXTILineConfig(EXTI_PortSourceGPIOA, EXTI_PinSource0);

SYSCFG_EXTILineConfig(EXTI_PortSourceGPIOE, EXTI_PinSource4);

EXTI_InitStructure.EXTI_Line = EXTI_Line0 | EXTI_Line4;

EXTI_InitStructure.EXTI_LineCmd = ENABLE;

EXTI_InitStructure.EXTI_Mode = EXTI_Mode_Interrupt;

EXTI_InitStructure.EXTI_Trigger = EXTI_Trigger_Falling;

EXTI_Init(&EXTI_InitStructure);

NVIC_InitStructure.NVIC_IRQChannel = EXTI0_IRQn;

NVIC_InitStructure.NVIC_IRQChannelCmd = ENABLE;

NVIC_InitStructure.NVIC_IRQChannelPreemptionPriority = 0x01;

NVIC_InitStructure.NVIC_IRQChannelSubPriority = 0x01;

NVIC_Init(&NVIC_InitStructure);

NVIC_InitStructure.NVIC_IRQChannel = EXTI4_IRQn;

NVIC_InitStructure.NVIC_IRQChannelPreemptionPriority = 0x01;

NVIC_Init(&NVIC_InitStructure);

}

void EXTI4_IRQHandler(void)

{

EXTI_ClearITPendingBit(EXTI_Line4);

led_count=0;

key_state+=1;

if(key_state==4)

key_state=0;

if(key_state==0)

key_state=1;

}

void EXTI0_IRQHandler(void)

{

EXTI_ClearITPendingBit(EXTI_Line4);

}

exit.h

#ifndef __EXTI_H

#define __EXTI_H

#include <stm32f4xx.h>

void EXTIx_Init(void);

#endif

timer.c

#include "timer.h"

#include "led.h"

void TIM3_Int_Init(u16 arr,u16 psc)

{

TIM_TimeBaseInitTypeDef TIM_TimeBaseInitStructure;

NVIC_InitTypeDef NVIC_InitStructure;

RCC_APB1PeriphClockCmd(RCC_APB1Periph_TIM3,ENABLE);

TIM_TimeBaseInitStructure.TIM_Period = arr;

TIM_TimeBaseInitStructure.TIM_Prescaler=psc;

TIM_TimeBaseInitStructure.TIM_CounterMode=TIM_CounterMode_Up;

TIM_TimeBaseInitStructure.TIM_ClockDivision=TIM_CKD_DIV1;

TIM_TimeBaseInit(TIM3,&TIM_TimeBaseInitStructure);

TIM_ITConfig(TIM3,TIM_IT_Update,ENABLE);

TIM_Cmd(TIM3,ENABLE);

NVIC_InitStructure.NVIC_IRQChannel=TIM3_IRQn;

NVIC_InitStructure.NVIC_IRQChannelPreemptionPriority=0x00;

NVIC_InitStructure.NVIC_IRQChannelSubPriority=0x03;

NVIC_InitStructure.NVIC_IRQChannelCmd=ENABLE;

NVIC_Init(&NVIC_InitStructure);

}

timer.h

#ifndef _TIMER_H

#define _TIMER_H

#include "sys.h"

void TIM3_Int_Init(u16 arr,u16 psc);

#endif